Flightradar24 Setup Guide

Raspberry Pi Model B Plus — Raspbian Bookworm

RTL-SDR / Edimax EW-7811Un WiFi Dongle / FR24 Feed

# 1. Overview

This guide describes the complete setup of a Flightradar24 ADS-B feeder on a Raspberry Pi Model B Plus (ARMv6/BCM2835) running Raspbian Bookworm. It covers WiFi configuration, fr24feed and dump1090 installation, and the local web map interface.

Architecture:

- dump1090-mutability — reads from RTL-SDR dongle, writes JSON, serves Beast TCP on port 30005

- fr24feed — connects to dump1090 via Beast TCP on port 30005, feeds data to Flightradar24

- lighttpd — serves the local gmap.html flight map web interface

⚠️  The Raspberry Pi Model B Plus uses ARMv6. Only fr24feed version 1.0.55-1 or later supports ARMv6. Always pin the package after install to prevent auto-upgrades breaking it.

# 2. Prerequisites

- Raspberry Pi Model B Plus with fresh Raspbian Bookworm SD card

- Edimax EW-7811Un WiFi USB dongle (RTL8188CUS chip)

- RTL-SDR USB dongle with ADS-B antenna

- Temporary wired ethernet connection for initial setup

- Your Flightradar24 sharing key (fr24key)

- Your WiFi SSID and password

# 3. WiFi Configuration

The Edimax EW-7811Un dongle uses the RTL8188CUS chip which has issues with WPA3 and power management on Bookworm. Follow these steps carefully.

## 3.1 Verify the dongle is detected

lsusb

You should see: Edimax Technology Co., Ltd EW-7811Un 802.11n Wireless Adapter [Realtek RTL8188CUS]

## 3.2 Scan for available networks

sudo nmcli device wifi rescan && sleep 3 && sudo nmcli device wifi list

Your network should appear in the list. If it does not, run the rescan command again.

## 3.3 Create a WPA2-forced connection profile

The RTL8188CUS cannot handle WPA3 negotiation. Force WPA2 explicitly:

sudo nmcli connection add type wifi ifname wlan0 con-name "YourSSID" ssid "YourSSID" \

wifi-sec.key-mgmt wpa-psk \

wifi-sec.psk "YourPassword" \

wifi-sec.proto rsn \

wifi-sec.pairwise ccmp \

wifi-sec.group ccmp

sudo nmcli connection up "YourSSID"

⚠️  If connection fails with 'network not found', try: sudo nmcli radio wifi off && sudo nmcli radio wifi on — then retry the connection up command.

## 3.4 Disable WiFi power saving

Without this, the WiFi connection will drop after a few minutes and never reconnect:

sudo nano /etc/NetworkManager/conf.d/wifi-powersave-off.conf

Add the following content:

[connection]

wifi.powersave = 2

sudo systemctl restart NetworkManager

## 3.5 Verify WiFi is working

ip a s wlan0

ping -c 4 8.8.8.8 -I wlan0

wlan0 should have an IP address and ping should show 0% packet loss.

# 4. Blacklist RTL-SDR Kernel Modules

The default DVB kernel modules will grab the RTL-SDR dongle and prevent fr24feed from using it. Blacklist them permanently:

sudo nano /etc/modprobe.d/blacklist-rtl.conf

Add the following lines:

blacklist dvb_usb_rtl28xxu

blacklist rtl2832

blacklist rtl2830

blacklist e4000

💡  The WiFi dongle uses rtl8192cu which is a different driver — do NOT blacklist it.

Reboot to apply:

sudo reboot

Verify after reboot that the modules are not loaded:

lsmod | grep -E 'rtl2832|dvb_usb_rtl'

This should return no output.

# 5. Install fr24feed

## 5.1 Add the FR24 repository

⚠️  Use HTTP (not HTTPS) — the FR24 repo has a TLS certificate mismatch that blocks HTTPS.

sudo bash -c "echo 'deb [trusted=yes] http://repo.feed.flightradar24.com flightradar24 raspberrypi-stable' > /etc/apt/sources.list.d/fr24feed.list"

sudo apt-get update

## 5.2 Install fr24feed

sudo apt-get install fr24feed

## 5.3 Pin fr24feed immediately

This prevents auto-upgrades from replacing the ARMv6-compatible binary with an incompatible one:

sudo apt-mark hold fr24feed

## 5.4 Verify the binary works

fr24feed --version

Should output: 1.0.55-1 (or later). If it shows 'Segmentation fault', the wrong architecture package was installed.

# 6. Install dump1090-mutability

## 6.1 Install the package

sudo apt-get install dump1090-mutability

## 6.2 Create the symlink for fr24feed

fr24feed looks for dump1090 at /usr/lib/fr24/dump1090 but it is installed at /usr/bin/dump1090-mutability:

sudo ln -s /usr/bin/dump1090-mutability /usr/lib/fr24/dump1090

## 6.3 Enable and configure dump1090

sudo nano /etc/default/dump1090-mutability

Make the following changes:

- Set START_DUMP1090="yes"

- Set NET_BIND_ADDRESS="0.0.0.0" (allows access from your network)

Or use sed to do it in one step:

sudo sed -i 's/START_DUMP1090="no"/START_DUMP1090="yes"/' /etc/default/dump1090-mutability

sudo sed -i 's/NET_BIND_ADDRESS="127.0.0.1"/NET_BIND_ADDRESS="0.0.0.0"/' /etc/default/dump1090-mutability

Verify the changes:

grep START_DUMP1090 /etc/default/dump1090-mutability

grep NET_BIND_ADDRESS /etc/default/dump1090-mutability

## 6.4 Enable and start dump1090

sudo systemctl enable dump1090-mutability

sudo systemctl start dump1090-mutability

sudo systemctl status dump1090-mutability

Verify it is listening on the expected ports:

sudo ss -tlnp | grep dump1090

You should see ports 30001, 30002, 30003, 30004, 30005, 30104 all listening.

# 7. Configure fr24feed

## 7.1 Create the configuration file

sudo nano /etc/fr24feed.ini

Add the following content (replace YOUR_FR24KEY with your actual key):

receiver="beast-tcp"

host="127.0.0.1:30005"

fr24key="YOUR_FR24KEY"

bs="no"

raw="no"

logmode="0"

logpath="/var/log/fr24feed"

mlat="no"

mlat-without-gps="no"

bind-interface="0.0.0.0"

⚠️  The receiver=beast-tcp mode makes fr24feed connect to the already-running dump1090 instance rather than spawning its own. This is required for the local web map to work correctly.

## 7.2 Enable and start fr24feed

sudo systemctl enable fr24feed

sudo systemctl start fr24feed

sudo systemctl status fr24feed

Verify fr24feed is receiving and sending aircraft data:

sudo journalctl -u fr24feed -n 20 --no-pager

Look for lines like: [feed][i]sent 35,0 AC — this confirms data is flowing.

# 8. Install the Web Map Interface

## 8.1 Install lighttpd

sudo apt-get install lighttpd

## 8.2 Enable the dump1090 module

sudo lighty-enable-mod dump1090

sudo service lighttpd force-reload

## 8.3 Verify the web interface is working

Open a browser and go to:

http://<your-pi-ip>/dump1090/gmap.html

Aircraft should be visible on the map with no error message.

💡  Find your Pi's IP address with: ip a s wlan0

# 9. Final Verification

After a reboot, verify all services started correctly:

sudo reboot

After reboot:

sudo systemctl status fr24feed

sudo systemctl status dump1090-mutability

sudo systemctl status lighttpd

ps aux | grep dump1090

ip a s wlan0

ping -c 4 8.8.8.8 -I wlan0

Expected results:

- fr24feed: active (running)

- dump1090-mutability: active (running)

- lighttpd: active (running)

- Only ONE dump1090 process visible in ps aux

- wlan0 has an IP address

- Ping shows 0% packet loss

Also check the FR24 dashboard and local map:

- http://<your-pi-ip>:8754/ — FR24 feeder dashboard showing aircraft count

- http://<your-pi-ip>/dump1090/gmap.html — local map with no error message

# 10. Known Issues & Solutions

## fr24feed segfaults on --version

Cause: Wrong architecture package (armhf) installed on ARMv6 Pi.

Solution: Ensure fr24feed version 1.0.55-1 or later is installed. Contact FR24 support if the repo only provides 1.0.55-0.

## WiFi connects but drops after ~10 minutes

Cause: WiFi power saving mode enabled.

Solution: Create /etc/NetworkManager/conf.d/wifi-powersave-off.conf with wifi.powersave = 2 (see section 3.4).

## WiFi shows 'network not found' despite network being visible in scan

Cause: WPA3 negotiation failure with RTL8188CUS chip, or driver state issue.

Solution: Force WPA2 using the nmcli connection profile (see section 3.3), or toggle WiFi radio off/on: sudo nmcli radio wifi off && sudo nmcli radio wifi on

## gmap.html shows 'Problem fetching data from dump1090'

Cause: fr24feed is spawning its own dump1090 instance (without --net) instead of using the standalone one.

Solution: Ensure fr24feed.ini uses receiver="beast-tcp" and host="127.0.0.1:30005", and that the standalone dump1090-mutability service is running.

## dump1090-mutability shows 'disabled' and won't start

Cause: START_DUMP1090 is set to "no" in /etc/default/dump1090-mutability.

Solution: Run: sudo sed -i 's/START_DUMP1090="no"/START_DUMP1090="yes"/' /etc/default/dump1090-mutability

## fr24feed shows 'Connection refused' connecting to dump1090

Cause: fr24feed started before dump1090-mutability was ready, or incorrect host/port in fr24feed.ini.

Solution: Ensure host="127.0.0.1:30005" (port included in host string) in fr24feed.ini, then restart fr24feed after dump1090 is running.

# 11. Important Files to Back Up

| File | Purpose |
| --- | --- |
| /etc/fr24feed.ini | FR24 feeder config including your fr24key |
| /etc/default/dump1090-mutability | dump1090 startup configuration |
| /etc/modprobe.d/blacklist-rtl.conf | RTL-SDR kernel module blacklist |
| /etc/NetworkManager/system-connections/ | WiFi connection profiles |
| /etc/NetworkManager/conf.d/wifi-powersave-off.conf | WiFi power saving config |

# 12. Quick Reference Commands

### Check all service statuses

sudo systemctl status fr24feed dump1090-mutability lighttpd

### View fr24feed live log

sudo journalctl -fu fr24feed

### Verify only one dump1090 instance

ps aux | grep dump1090

### Check listening ports

sudo ss -tlnp | grep -E 'dump1090|lighttpd'

### Check WiFi status

ip a s wlan0 && nmcli connection show --active

### Restart all services

sudo systemctl restart dump1090-mutability && sudo systemctl restart fr24feed && sudo systemctl restart lighttpd